457.568 Statistical Hydrology, Fall 2022

R Code Training Assignment #1 (Probabilistic Modeling)

Due before the lecture of Oct-18, 2022

<Notice> Include responses to every question below, and make sure your final assignment is well organized (i.e., numbered responses, full sentences, code attached at the end). This makes it easier to grade & to give partial credits as well.

- (R Loop Exercise) You are about to print the following message using for loops in R.

print(paste("The year is", 2010)) "The year is 2010" print(paste("The year is", 2011)) "The year is 2011" print(paste("The year is", 2012)) "The year is 2012" print(paste("The year is", 2013)) "The year is 2013" print(paste("The year is", 2014)) "The year is 2014" print(paste("The year is", 2015)) "The year is 2015"

  - Using a for loop, create a command that produces the same results with the above example.

  - Again, use two for loops to create a command that produces the results with both month and the year. Assume that there are only three months: “Jan”, “Feb”, and “Mar.” Your results should start from Jan-2010 and finish with Mar-2015.

- 윤진아씨는 빵집을 운영하고 있다. 하루 평균 20명의 방문자가 방문하고, 매일 방문하는 방문자의 수는 Poisson 분포를 따른다고 가정한다 또한, 방문자는 재고가 없지 않는 이상 케이크를 무조건 구매하며, 하루 최소 방문자는 1명, 최대 방문자의 수는 40명이라고 가정한다. 케이크 하나의 가격은 35,000원이고, 만드는데 드는 비용은 25,000원이다. 윤진아씨는 혼자 빵집을 운영하기 때문에 영업 전에 미리 케이크를 준비해 놓으며, 당일에 팔지 못하는 케이크는 폐기한다고 한다.

- 매일 케이크를 얼마나 준비하는 것이 기대 이윤 (즉, 총 수입과 총 비용의 차이)을 극대화할 수 있는 방안인가?

- 손님이 방문하였으나 케이크 재고가 부족하여 팔지 못하면 대신 기념품을 제공하고, 해당 기념품의 제작 비용은 20,000원이라고 하자. 기대 이윤을 극대화하기 위해 준비해야 하는 케이크의 수는 어떻게 바뀌는가?

- Conduct the probabilistic model analysis using the data provided as “OwegoCreekFlow-1.csv” following the steps below.

  - Load the data into R using the read.csv() function. Then, use the aggregate() function to average the monthly flow data for each calendar month, generating 12 monthly average values. Create a figure of this monthly hydrograph. Determine and report which month has the lowest average monthly flow. This will be the ‘design month’: by examining water demand and supply conflicts during the driest month, our analysis will consider the most constraining time of year for water withdrawals.

  - Create a new variable in R (‘owego.min.month’) that only contains the data for the design month. This variable should be a vector that has 48 elements, one for each year of the record.

  - Plot a histogram of the data in owego.min.month using the hist() function. Briefly comment on the shape of the distribution. Is it symmetric? Is it skewed?

  - Determine an appropriate distribution with which to model the monthly flows for the design month. Fit the normal, log-normal, and gamma distributions to these data using maximum likelihood. You can use functions in R designed for MLE, such as fitdistr() in the MASS package.

  - Calculate and report the log likelihood using the MLE parameter estimates for each distribution. This is included in the output of the fitdistr() function. Report the fitted parameters and log-likelihood value for each model.

______________________________________________________________________________

  - Calculate and report the AIC for each model. Which distribution will you choose to model these data?

  - The AIC values above give a sense of which distribution of the three (normal, lognormal, and gamma) fits the data the best relative to the other distributions, but it doesn’t tell you if the fit is good in an absolute sense. For the model that you choose in F, create a P-P plot to check the fit directly. Comment on the goodness-of-fit given this plot.

  - To triple-check the fit of your chosen distribution, use a chi-squared test to conduct a hypothesis test. You can use the chisq.test() function. To conduct this test, first calculate the observed number of observations between (0-50 cfs), (50-100 cfs), and (>100 cfs). An easy way to do this is to combine the table and the cut functions:

obs.breaks <- cut(owego_min_month, breaks=c(0,50,100,Inf))

obs.counts <- table(obs.breaks)

cut() assigns each observation to a particular bin, while table() counts the total number of times observations fall into each of the three bins. Run the variables obs.breaks and obs.counts in the console to see this for yourselves.

In addition to the observed counts, you need to calculate the expected probabilities (exp.probs) that the data would fall into each bin, assuming the distribution you chose was a good fit to the data (hint: to do this, you can make use of the cdf function for your chosen distribution, e.g., pgamma() if you chose the gamma distribution). Note that the chisq.test() takes expected probabilities, not expected counts, although exp.probs should be a vector of the same length as the vector obs.counts. Report the X-squared statistic, degrees of freedom, and the p-value from the chi-square test returned by chisq.test(x=obs.counts, p=exp.probs). How do you interpret the results of this test?